## 2016 Consumer Confidence Report

| Water System Name: | Lake Elizabeth Mutual Water Company | Report Date: | 6/2017 |
| --- | --- | --- | --- |

We test the drinking water quality for many constituents as required by state and federal regulations.  This report shows the results of our monitoring for the period of January 1 - December 31, 2016 and also include some earlier monitoring data as required. We test for many more constituent in our water and some constituents not found, but may be of interest to you, are reported (ex. coliforms).

Based on the drinking water quality we gathered the past year, your tap water met all USEPA and State water health standards.

Este informe contiene información muy importante sobre su agua potable.  Tradúzcalo ó hable con alguien que lo entienda bien.

| Type of water source(s) in use: |  | Surface Water: State Water Project (Aqueduct) Ground Water: Wells |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name & general location of source(s): |  |  | Surface Water: Willow Turnout, Munz Ranch Road |  |  |  |  |  |
| Ground Water: Brookwood Well and Well #2 which are within our service area |  |  |  |  |  |  |  |  |
| Drinking Water Source Assessment information: |  |  |  |  |  |  |  |  |
| An assessment of drinking water source (Brookwood Well) for the Water System was completed in April 2009. It was conducted by Lake Elizabeth Mutual Water Company and California State Department of Public Health, predecessor to the State Water Control Board. The source is most vulnerable to activities associated with contaminants; septic systems. Other vulnerable storage tanks, housing and storm drains. A copy is available at the Water Company office and at California State Department of Public Health Office: 500 North Central Ave., Glendale, CA 91203. Surface water assessment can be viewed at http://www.avek.org/index/cfm?fuseaction=menu&menu-id=5008 or at the office of Antelope Valley-East Kern Water Agency at 6500 W. Ave. N, Palmdale CA, 93551. |  |  |  |  |  |  |  |  |
| Time and place of regularly scheduled board meetings for public participation: |  |  |  |  |  |  | The Annual Shareholder’s Meeting |  |
| is the second Tuesday in June at 6:00 PM at 14960 Elizabeth Lake Rd., Elizabeth Lake, CA 93532. |  |  |  |  |  |  |  |  |
| For more information, contact: | Lake Elizabeth Mutual Water Company |  |  |  |  | Phone: |  | ( 661 ) 724 - 1806 |
| TERMS USED IN THIS REPORT |  |  |  |  |  |  |  |  |
| Maximum Contaminant Level (MCL): The highest level of a contaminant that is allowed in drinking water. Primary MCLs are set as close to the PHGs (or MCLGs) as is economically and technologically feasible. Secondary MCLs are set to protect the odor, taste, and appearance of drinking water. Maximum Contaminant Level Goal (MCLG): The level of a contaminant in drinking water below which there is no known or expected risk to health. MCLGs are set by the U.S. Environmental Protection Agency (USEPA). Public Health Goal (PHG): The level of a contaminant in drinking water below which there is no known or expected risk to health. PHGs are set by the California Environmental Protection Agency. Maximum Residual Disinfectant Level (MRDL): The highest level of a disinfectant allowed in drinking water. There is convincing evidence that addition of a disinfectant is necessary for control of microbial contaminants. Maximum Residual Disinfectant Level Goal (MRDLG): The level of a drinking water disinfectant below which there is no known or expected risk to health. MRDLGs do not reflect the benefits of the use of disinfectants to control microbial contaminants. Primary Drinking Water Standards (PDWS): MCLs and MRDLs for contaminants that affect health along with their monitoring and reporting requirements, and water treatment requirements. |  |  |  |  | Secondary Drinking Water Standards (SDWS): MCLs for contaminants that affect taste, odor, or appearance of the drinking water. Contaminants with SDWSs do not affect the health at the MCL levels. Treatment Technique (TT): A required process intended to reduce the level of a contaminant in drinking water. Regulatory Action Level (AL): The concentration of a contaminant which, if exceeded, triggers treatment or other requirements that a water system must follow. Variances and Exemptions: State Board permission to exceed an MCL or not comply with a treatment technique under certain conditions. Level 1 Assessment: A Level 1 assessment is a study of the water system to identify potential problems and determine (if possible) why total coliform bacteria have been found in our water system. Level 2 Assessment: A Level 2 assessment is a very detailed study of the water system to identify potential problems and determine (if possible) why an E. coli MCL violation has occurred and/or why total coliform bacteria have been found in our water system on multiple occasions. ND: not detectable at testing limit ppm: parts per million or milligrams per liter (mg/L) ppb: parts per billion or micrograms per liter (µg/L) ppt: parts per trillion or nanograms per liter (ng/L) ppq: parts per quadrillion or picogram per liter (pg/L) pCi/L: picocuries per liter (a measure of radiation) |  |  |  |

The sources of drinking water (both tap water and bottled water) include rivers, lakes, streams, ponds, reservoirs, springs, and wells.  As water travels over the surface of the land or through the ground, it dissolves naturally-occurring minerals and, in some cases, radioactive material, and can pick up substances resulting from the presence of animals or from human activity.

Contaminants that may be present in source water include:

- Microbial contaminants, such as viruses and bacteria that may come from sewage treatment plants, septic systems, agricultural livestock operations, and wildlife.

- Inorganic contaminants, such as salts and metals, that can be naturally-occurring or result from urban storm water runoff, industrial or domestic wastewater discharges, oil and gas production, mining, or farming.

- Pesticides and herbicides, that may come from a variety of sources such as agriculture, urban storm water runoff, and residential uses.

- Organic chemical contaminants, including synthetic and volatile organic chemicals that are by-products of industrial processes and petroleum production, and can also come from gas stations, urban storm water runoff, agricultural application, and septic systems.

- Radioactive contaminants, that can be naturally-occurring or be the result of oil and gas production and mining activities.

In order to ensure that tap water is safe to drink, the USEPA and the State Water Resources Control Board (State Board) prescribe regulations that limit the amount of certain contaminants in water provided by public water systems.  The U.S. Food and Drug Administration regulations and California law also establish limits for contaminants in bottled water that provide the same protection for public health.

Tables 1, 2, 3, 4, 5, and 6 list all of the drinking water contaminants that were detected during the most recent sampling for the constituent. As required, we test for many more constituent in our water.  Some constituents not detected and may be of interest to you are included here (ex. coliforms).   The presence of detected contaminants in the water does not necessarily indicate that the water poses a health risk.  The State Board allows us to monitor for certain contaminants less than once per year because the concentrations of these contaminants do not change frequently.  Some of the data, though representative of the water quality, are more than one year old.  Any violation of an AL, MCL, MRDL, or TT is asterisked.  Additional information regarding the violation is provided later in this report.

| Table 1 – MICROBIOLOGICAL |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Microbiological Contaminants | Highest No. of Positive Samples |  | Total No. of Positive Samples | No. of months in violation |  | MCL |  |  | MCLG | Typical Sources of Contaminants |
| State Total Coliform Rule (or TCR), from 1/1/16 - 12/31/16: : |  |  |  |  |  |  |  |  |  |  |
| Total Coliform Bacteria Fecal coliform and E. coli | 0 (zero) (In any month) |  | 0 (zero) (the whole year) | 0 (zero) |  | One (1) positive monthly sample |  |  | 0 | Naturally present in the environment Human and animal fecal waste |
| Federal Revised TCR, (from 4/1/16 - 12/31/16): |  |  |  |  |  |  |  |  |  |  |
| E. coli |  |  | 0 (zero) (the whole year) | 0 (zero) |  | See note (a) below |  |  | 0 | Human and animal fecal waste |
| Note (a) Routine and repeat samples are total coliform-positive and either is E. coli-positive or system fails to take repeat samples following E. coli-positive routine sample or system fails to analyze total coliform-positive repeat sample for E. coli. |  |  |  |  |  |  |  |  |  |  |
| Table 2 – Lead and copper |  |  |  |  |  |  |  |  |  |  |
| Contaminants | Sample Date |  | No. of samples collected | 90th percentile level detected |  | No. sites exceeding AL | AL |  | PHG | Typical Sources of Contaminants |
| Lead (ppb) | 10/28/15 |  | 10 | 1.1 |  | 0 | 15 |  | 0.2 | Internal corrosion of household water plumbing systems; discharges from industrial manufacturers; erosion of natural deposits. |
| Copper (ppm) | 10/28/15 |  | 10 | .3 |  | 0 | 1.3 |  | 0.3 | Internal corrosion of household plumbing systems; erosion of natural deposits; leaching from wood preservatives. |
| TAble 3 – sodium, hardness, and other constituents’ SAMPLING RESULTS (No standards) |  |  |  |  |  |  |  |  |  |  |
| Chemical or Constituent (and reporting units) |  | Sample Date |  |  | Level Detected |  |  | Typical Source of Contaminant |  |  |
| Sodium (ppm) |  | 10/26/16 |  |  | 73 ppm |  |  | Salt present in the water and is generally naturally occurring |  |  |
| Hardness (ppm) |  | 10/26/16 |  |  | 95.9 ppm |  |  | Sum of polyvalent cations present in the water, generally magnesium and calcium, and are usually naturally occurring |  |  |
| Potassium (ppm), Surface Water |  | 10/26/16 |  |  | 3 ppm |  |  |  |  |  |
| Bicarbonate (ppm), Surface Water |  | 10/26/16 |  |  | 80 ppm |  |  |  |  |  |
| Calcium (Ca) (ppm), Surface Water |  | 10/26/16 |  |  | 17 ppm |  |  |  |  |  |
| Magnesium (ppm), Surface Water |  | 10/26/16 |  |  | 13 ppm |  |  |  |  |  |

| TAble 4 – detection of contaminants with Primary Drinking Water StandardS |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Chemical or Constituent (and reporting units) | Sample Date | Level Detected | MCL | PHG (MCLG) | Typical Source of Contaminant |
| Aluminum (ppb), Surface Water | 10/26/16 | 650 ppb | 1000 ppb | 600 | Erosion of natural deposits; residue from some surface water treatment processes |
| Arsenic (ppb), Surface Water | 10/26/16 | 3 ppb | 10 ppm | 0.004 | Erosion of natural deposits; runoff from orchards; glass and electronics production wastes |
| *Nitrate as N (Nitrogen), (ppm) Surface Water Well #2 Brookwood Well | 4 samples 10/26/16 3/24/16 | 0.8 (average of 4 samples) 7.3 ppm 4.2 | 10 ppm As N | 10 ppm As N | Runoff and leaching from fertilizer use; leaching from septic tanks and sewage; erosion of natural deposits |
| Notes: *Nitrate in drinking water at levels above 10 mg/L is a health risk for infants of less than six months of age. Such nitrate levels in drinking water can interfere with the capacity of the infant’s blood to carry oxygen, resulting in a serious illness; symptoms include shortness of breath and blueness of the skin. Nitrate levels above 10 mg/L may also affect the ability of the blood to carry oxygen in other individuals, such as pregnant women and those with certain specific enzyme deficiencies. If you are caring for an infant, or you are pregnant, you should ask advice from your health care provider. |  |  |  |  |  |

| TAble 5 – detection of contaminants with Secondary Drinking Water StandardS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Chemical or Constituent (and reporting units) |  |  |  | Sample Date |  |  | Level Detected |  |  | MCL |  |  |  | Typical Source of Contaminant |  |  |
| Chloride (ppm), Surface Water |  |  |  | 10/26/16 |  |  | 125 ppm |  |  | *500 ppm |  |  |  | Runoff/leaching from natural deposits; seawater influence |  |  |
| Manganese (ppb), Surface Water |  |  |  | 10/26/16 |  |  | 20 ppb |  |  | 50 ppb |  |  |  | Leaching from natural deposits |  |  |
| Sulfate (ppm), Surface Water |  |  |  | 10/26/16 |  |  | 27.8 ppm |  |  | *500 ppm |  |  |  | Runoff/leaching from natural deposits; industrial wastes |  |  |
| Notes: *600 ppm is the maximum contaminant level in the short term only, for chloride and sulfate. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| TAble 6 –disinfectants/disinfectants by-products (D/DBP) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| D/DBP Contaminant | Sample Date |  | *Level Detected |  |  | MCL |  |  | PHG (MCLG) |  | Violation | Typical Source of Contaminant |  |  |  |  |
| TTHM (ppb) | 2016 |  | 52.70 and 67.20 |  |  | 80 |  |  | n/a |  | No | Byproduct of drinking water disinfection |  |  |  |  |
| HAA5 (ppb) | 2016 |  | 14.00 and 23.25 |  |  | 60 |  |  | n/a |  | No | Byproduct of drinking water disinfection |  |  |  |  |
| Notes: *Level detected at the Locational Running Annual Average (LRAA) during the fourth quarter of 2016. Data provided are for two sample sites. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| TAble 7 – radioactive |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Radioactive Contaminant |  | Sample Date |  |  |  |  |  | Level Detected | MCL |  | PHG (MCLG) | Violation |  |  | Typical Source of Contaminant |  |
| Gross Alpha (pCi/L) |  | Surface water: 10/28/15 Brookwood Well: 8/9/12 Well 2 Standby: 4/17/14 |  |  |  |  |  | 4.80 5.52 1.67 | 15 |  | (0) | No |  |  | Erosion of natural deposits |  |
| Uranium (pCi/L) |  | Surface water: 10/28/15 Brookwood Well: 8/9/12 Well 2 Standby: 4/17/14 |  |  |  |  |  | 0.317 4.76 1.88 | 20 |  | 0.43 | No |  |  | Erosion of natural deposits |  |
| Radium-226 (pCi/L) |  | Surface water: 10/28/15 Brookwood Well: 8/9/12 Well 2 Standby: 4/17/14 |  |  |  |  |  | 0.000 0.355 0.000 | 5 (for combined *Ra-226 and Ra-228) |  | 0.05 pCi/L | No |  |  | Erosion of natural deposits |  |
| Radium-228 (pCi/L) |  | Surface water: 10/28/15 Brookwood Well: 8/9/12 Well 2 Standby: 4/17/14 |  |  |  |  |  | 0.014 0.000 0.000 |  |  | 0.019 pCi/L | No |  |  | Erosion of natural deposits |  |
| Notes: *Ra-226 and Ra-228 are the same as Radium-226 and Radium-228, respectively.. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| TAble 8 –STATE REGULATED CONTAMINANTS with no mclS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Chemical or Constituent (and reporting units) |  |  |  |  | Sample Date |  |  |  | Level Detected |  |  |  | Notification Level |  |  |  |
| Boron (ppb), Surface water |  |  |  |  | 10/26/16 |  |  |  | 100 ppb |  |  |  | 1,000 ppb |  |  |  |
| Vanadium (ppb), Surface Water |  |  |  |  | 10/26/16 |  |  |  | 4 ppb |  |  |  | 50 ppb |  |  |  |

Additional General Information on Drinking Water

Drinking water, including bottled water, may reasonably be expected to contain at least small amounts of some contaminants.  The presence of contaminants does not necessarily indicate that the water poses a health risk. More information about contaminants and potential health effects can be obtained by calling the USEPA’s Safe Drinking Water Hotline (1-800-426-4791).

Some people may be more vulnerable to contaminants in drinking water than the general population.  Immuno-compromised persons such as persons with cancer undergoing chemotherapy, persons who have undergone organ transplants, people with HIV/AIDS or other immune system disorders, some elderly, and infants can be particularly at risk from infections. These people should seek advice about drinking water from their health care providers. USEPA/Centers for Disease Control (CDC) guidelines on appropriate means to lessen the risk of infection by Cryptosporidium and other microbial contaminants are available from the Safe Drinking Water Hotline (1-800-426-4791).

Lead-Specific Language for Community Water Systems:  If present, elevated levels of lead can cause serious health problems, especially for pregnant women and young children.  Lead in drinking water is primarily from materials and components associated with service lines and home plumbing.  Lake Elizabeth Mutual Water Company is responsible for providing high quality drinking water, but cannot control the variety of materials used in plumbing components.  When your water has been sitting for several hours, you can minimize the potential for lead exposure by flushing your tap for 30 seconds to 2 minutes before using water for drinking or cooking. [Optional: If you do so, you may wish to collect the flushed water and reuse it for another beneficial purpose, such as watering plants.] If you are concerned about lead in your water, you may wish to have your water tested.  Information on lead in drinking water, testing methods, and steps you can take to minimize exposure is available from the Safe Drinking Water Hotline (1-800-426-4701) or at http://www.epa.gov/lead.

For Water Systems Providing Ground Water as a Source of Drinking Water

| TAble 9 – SAMPLING RESULTS SHOWING feCal indicator-positive ground water source samples |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Microbiological Contaminants (complete if fecal-indicator detected) | Total No. of Detections | Sample Dates | MCL [MRDL] | PHG (MCLG) [MRDLG] | Typical Source of Contaminant |
| E. coli Treatment Plant Well #2 Brookwood Well | (In the year) 0 0 0 | Weekly Weekly Monthly | 0 0 0 | (0) (0) (0) | Human and animal fecal waste |

For Systems Providing Surface Water as a Source of Drinking Water

| Table 10 - sampling results showing TREATMENT OF SURFACE WATER SOURCES |  |
| --- | --- |
| Treatment Technique (a) (Type of approved filtration technology used) | Conventional Filtration |
| Turbidity Performance Standards (b) (that must be met through the water treatment process) | Turbidity of the filtered water must: 1 – Be less than or equal to 0.3 NTU in 95% of measurements in a month. 2 – Not exceed 1.0 NTU for more than eight consecutive hours. 3 – Not exceed 5.0 NTU at any time. |
| Lowest monthly percentage of samples that met Turbidity Performance Standard No. 1. | 99.9% |
| Highest single turbidity measurement during the year | 1.42 NTU |
| Number of violations of any surface water treatment requirements | 0 (NONE) |

(a)	A required process intended to reduce the level of a contaminant in drinking water.

(b)	Turbidity (measured in NTU) is a measurement of the cloudiness of water and is a good indicator of water quality and filtration performance.  Turbidity results which meet performance standards are considered to be in compliance with filtration requirements.